Ryan Laur, Benjamin Wheeler

# Lab 2 Report

Reconfigurable Computing 2

# Bugs with done signal

## Done cleared too early

Preconditions:

- DUT in done state (done == 1)

- Testbench restarted DUT via go == 1

Violation:

- Done not cleared on following cycle (violates design specification)

Detected in fib_tb.sv line 100:

// if go and done are both asserted, done should be cleared on the next cycle

assert property (@(posedge bfm.clk) disable iff (bfm.rst) bfm.go && bfm.done |=> !bfm.done)

else $error("Time %0t [Assert Property]: Done=1, go=1, done not cleared next cycle.", $time);

Modelsim errors:

## Done cleared too late

Preconditions:

- DUT in done state (done == 1)

- Testbench restarted DUT via go == 1

Violation:

- Done took two cycles to clear instead of one.

Detected in fib_tb.sv line 100 (see assertion property for first error).

# Infinite loop bugs

## Max number as an input causes state machine to never finish.

Preconditions:

- Circuit fed input of max number (e.g., WIDTH’(1’b1))

  - This input is the largest number the circuit can handle for its specified width.

Violation:

- Circuit never asserts done, gets caught in infinite loop

Detected via a watchdog (monitor.svh line 131) that quits the simulation after done is not seen for 100k cycles.

# Bug: Result register changes when circuit done

Preconditions:

- Circuit reaches done state

- Circuit asserts done signal

Violation:

- Result register changes as done is asserted, which should not happen.

Detected in fib_tb.sv line 120, 123:

// upon completion, (ie done = 1), result and overflow retain their values until circuit is restarted

assert property (@(posedge bfm.clk) disable iff (bfm.rst) bfm.done && $stable(bfm.done) |-> $stable(bfm.result))

else $error("Time %0t [Assert Property]: Done=1, result not stable.", $time);

// upon completion, (ie done = 1), result and overflow retain their values until circuit is restarted

assert property (@(posedge bfm.clk) disable iff (bfm.rst) bfm.done && $stable(bfm.done) |-> $stable(bfm.overflow))

else $error("Time %0t [Assert Property]: Done=1, overflow not stable.", $time);

# Bug: Circuit reacts to changes on the input

Preconditions:

- Circuit is started with go

- Input n is changed before the circuit finishes (before done == 1)

Violation:

- Circuit does not produce correct output

Discovered via scoreboard logic that ensures l, x, and y registers have correct values compared to the model function. Since they were different, this means the DUT is stopping before it should and thus producing incorrect output.

# Overflow bugs

Breaks at n = 0x3f